Classic Boost
The Classic Boost is a dirty transistor boost based on the EHX LPB1™.

Bill of materials

| Resistors | Diode |
| --- | --- |
| R1 1Meg | D1 1n4001 |
| R2 10K |  |
| R3 560R | Transistor |
| R4 100k | U1 2n5088 |
| R5 1Meg |  |
|  | Potentiometer |
| Capacitors | Volume 100ka (9mm Log Pot) |
| C1 100nf (104) | Tone 100kb (9mm Linear Pot) |
| C2 10nf (103) |  |
| C3 100nf (104) |  |

1590a
The board spacing will fit a 1590a enclosure.  You can mount 9mm pots directly to the board.

Tone mod

Changing the values of c1 and c2 will change the frequency range of the Tone Control. Also note c3 will affect the bass output of the circuit as well (increase for more bass, decrease for less).  I’ve found the best combination for this set up is as listed above.  Socket these 3 caps and experiment / adjust to taste.