<Effect Name>

Designed By Erik (DBE)

<Effect Description><What do you get when a One Knob Fuzz collides with a Big Muff Pi Tone Stack?  Ok, minds out of the gutter…The Big Knob of course!  I’ve always felt the One Knob Fuzz could use a tone control for versatility as well as experimentation.  The Big Muff Pi won the deal, it marries seamlessly with the One Knob Fuzzes expressive output.  16mm pot spacing and silicon transistors make this project a good contender for a silicon fuzz with “bang for buck”.>

<Bill of materials>

| Resistors | Capacitors |
| --- | --- |
| R1 1M | C1 100nf |
| R2 10k | C2 220pf |
| R3 82R (see notes) | C3 10nf |
| R4 2k2 | C4 10uf |
| R5 150k | C5 22nf |
| R6 1k | C6 220nf |
| R7 39k | C7 220nf |
| R8 47k |  |
| R9 430k | Transistors |
| R10 100k | Q1 2n3904 (see notes) |
| R11 15k | Q2 2n3904 (see notes) |
| R12 3k3 | Q3 2n5088 |
| Potentiometer | Diodes |
| Volume 100ka (16mm) | D1 1n4001 |
| Tone 100kb (16mm) |  |

PCB Spacing
The Big Knob PCB is spaced for 1590B sized enclosures or larger

Pot Spacing

The Big Knob’s pcb mounted potentiometers are spaced for Alpha 16mm potentiometers.

Modifications

Following is a couple of worthwhile modifications that I discovered expermenting with the Big Knob

Transistors

This ones obvious.  Q1 and Q2 form the gain stages of this silicon fuzz.  Increasing and decreasing the gains of these transistors will effect the tonal character of the fuzz.  In my experimentation a lot of NPN’s sounded good, I personally perferred the S9013’s though (available on my store) as they are relatively low gain. To intention mismatch gains (like a lot of fuzzes have), a 2n3904 in Q1 and a higher gain transistor in Q2 may increase sustain of the effect. The original circuit used BC107 and BC108 for Q1 and Q2 respectively, these can still be sourced.  Q3 is the recovery stage of the big muff and wont have much effect on tone shaping.

R3

For most settings the Big Muff Tone Stack should be at unity or greater.  However as the big muff pi is so reknowned for sucking the volume out of your signal you could consider socketing R3 and increasing its value for more output volume.  Resistor R3 controls the voltage to the collector of Q2 as well as the lpb1 style recovery stage booster on the output.  Replacing this resistor with a higher value can have an effect on the biasing of the circuit.  A 1k trimpot could be used for adjustment. Be warned a significant volume increase will occur when the trimpot is cranked.

Offboard Wiring Diagram
Using a non-switched Miniature DC Jacks and 2 Mono Jacks