Hydra Fuzz
Based on the Sola Sound Tonebender™ 3-knob

Bill of materials

| Resistors | Capacitors |
| --- | --- |
| R1 1M | C1 22uf |
| R2 47K | C2 100nf (104) |
| R3 220K | C3 220pf |
| R4 10K | C4 100nf (104) |
| R5 10K | C5 10uf |
| R6 3.3K | C6 100nf (104) |
| R7 10k | C7 2.2nf (222) |
| R8 18k (or Trim) | C8 10uf |
| R9 10k | C9 100uf |
| R10 10k |  |
| R11 220k (or Jumper) | Transistor |
|  | Q1 PNP Germanium Transistor |
| Diode | Q2 PNP Germanium Transistor |
| D1 1n4001 | Q3 PNP Germanium Transistor |
| D2 1n270 |  |
|  | Potentiometer |
| IC | Volume 100KB Lin |
| IC1 Charge Pump (TC1044scpa) | Tone 100KB Lin |
|  | Fuzz 100KB Lin |

1590b
The board spacing will fit a 1590b enclosure.  You can mount 9mm pots directly to the board.

Pot Tapers

Pot tapers of the original Sola Sound Tonebender™ used all linear potentiometers, even for the volume control.  I would definitely recommend a log pot for the volume control to avoid bunching.

Bias Mod

Replace the 18k resistor with a 50k trim to dial in some splatty misbiased fuzz sounds with your Hyrda Fuzz!  Wire in a pot instead for an easily adjustable bias control.

R11

Replacing R11 with a jumper will allow more output volume from the effect

Boxing up your effect

Watch my offboard wiring tutorial for information on wiring this effect inside an enclosure with LED, stompswitch, etc

http://www.youtube.com/watch?v=z6fpwU8RY_0