Smart ByPass

Design By Erik Vincent

Reliable, Durable, Regulated.  The Smart ByPass has been designed with a “smart” preprogrammed ATTINY chip at its core and covers almost everything you could want in a bypass solution.  This kit comes with everything you need (see BOM below) to build the Smart ByPass, including a preprogrammed chip, making its construction for you simply a matter of soldering.  Some of the many features of the Smart ByPass include:

- Reliable non-latching SPDT stomp E-Switch (rated at 30,000 cycles)

- Reliability of a relay bypass solution (analogue latching relay)

- Regulated (power filtered) 5v output

- Smarter reverse polarity protection for your pedals using PFETs instead of diodes

- Kit includes Preprogrammed attiny13a and all parts for easy assembly including the footswitch!

- The state of the relay is held for ~1/2 a second if power is disconnected. This is to prevent slight power interruptions that would otherwise cause the Smart Bypass relay to switch to bypassed mode.

Bill of materials

| Relay | Capacitor |
| --- | --- |
| L1 TQ2-L-5V | C1 1uf (film) |
|  | C2 1uf (film) |
| Stomp Switch | C3 100nf (film) |
| Mini Non-Latching SPDT Stomp Switch |  |
| IC | Transistor |
| U1 LP2950-50LPRE3 (68MR7CE) | Q1 VP2106N3-G |
| U2 ATTINY13A-PU |  |
| U3 G3VM-351A | Resistor |
|  | R1 1k5 |
| LED | R2 1k5 (LED CLR) |
| LED1 5mm Status LED | R3 10k |

Assembly / Soldering

- This is the top of the Smart ByPass PCB.

- This is the bottom of the PCB. Note the J1 Header

- First, solder the 1/4W resistors. The values for R1 and R2 are 1.5K. The value or R3 is 10K.

- Next, solder the P-Channel MOSFET, Q1.

- Next, solder the 5V Regulator, U1.

- Next, solder the 8-Pin socket onto U2. Let Bono and the Edge know.

- Next, solder the solid-state relay, U3

- Next, solder the small current latching relay, L1

- Next, solder the film capacitors, first with C3, then C1 and C2. The values of C1 and C2 are 1uF. The value of C3 is 0.1uF

- Next, solder up the stomp switch to SW1

- Next, solder the LED to LED1. It should stand off a little bit as to get it closer to where the stomp switch will be flush-mounted to the box of the pedal.

- Next, insert the programmed microcontroller. Make sure the markings are correct.

- Flip over to the bottom and note J1 and it’s labels.

J1 – Header connections

VIN – 9V coming in from battery or wall power
+9V OUT – protected 9V power. This connects to your effect board to provide it 9V power
+5V OUT – protected 5V power up to 100mA. This goes to your effect board if it needs 5V power
GND – this is for common ground components such as power ground, input/output jack ground, and effect board ground. Note, not all grounds may be utilized.
PEDAL OUT – this connects to the output jack’s output signal wire
PEDAL IN – this connects to the input jack’s input signal wire
FXO – this connects to the effect board’s output
FXI – this connects to the effect board’s input